Flexibility technologies in clean energy systems [8000]

Dawei Qiu1,2, Goran Strbac1 Cuo Zhang3, Zhao Yang Dong4 & Iain Staffell5

1Department of Electrical and Electronic Engineering, Imperial College London, London, UK. 2Department of Engineering, University of Exeter, Exeter, UK, 3School of Electrical and Computer Engineering, University of Sydney, Sydney, Australia. 4Department of Electrical Engineering, City University of Hong Kong, Kowloon Tong, Hong Kong Special Administrative Region of China. 5Centre for Environmental Policy, Imperial College London, London, UK.

Abstract [192]

As global energy systems transition toward net-zero emissions, maintaining real-time balance between supply and demand – known as energy system flexibility – has become a critical challenge. This challenge arises from the increasing penetration of variable renewable energies, the electrification of heat and transport sectors, and the decarbonization of gas infrastructure, all of which introduce unprecedented variability and uncertainty. In this Review, we examine the technologies and strategies that provide flexibility across clean energy systems, enabling reliable, efficient, and low-carbon energy delivery. We categorize flexibility into electricity, gas, and hydrogen systems, encompassing both supply- and demand-side mechanisms. Technologies such as clean dispatchable generation, large-scale storage, and grid interconnectors contribute to electricity system flexibility, while linepack, hydrogen electrolysis, and gas-fired generation provide balancing services in gas networks. Simultaneously, demand-side response is emerging as a scalable, distributed source of flexibility across the residential, commercial, industrial, and transport sectors, supported by advances in digitalization and decentralization. We further assess the global deployment status of these flexibility technologies, discuss the technical, economic, institutional, and social barriers that constrain their wider adoptions, and explore how a whole-system approach can unlock investment and maximize their role in clean energy transitions.

Introduction [676]

Global efforts to decarbonize energy systems are accelerating in response to the urgent need to mitigate climate change. Central to this transformation is the widespread electrification of end-use sectors, particularly heat and transport, alongside the rapid deployment of low-carbon energy sources. These include variable renewable energy resources (vRES) such as wind and solar, and emerging green fuels like hydrogen. By the end of 2024, global installed vRES capacity reached ~5.4 TW, while electrolyser capacity for green hydrogen exceeded 1.75 GW in 2023, with projections indicating a requirement for ~200 GW by 2030 to align with net-zero targets.

While these developments are critical, they are also fundamentally reshaping how energy is generated, delivered, and consumed. A defining characteristic of evolving energy systems is the sharp rise in variability and uncertainty, which poses new challenges for maintaining real-time supply-demand balance and system reliability. For example, in the United Kingdom (UK), growing reliance on wind power has introduced substantial variability. In December 2024, wind energy supplied over 50% of the country's electricity on about ten days; however, by 12 December, output suddenly dropped to just 6% during an extended period of low wind speeds, a phenomenon so-called 'wind droughts.' In Australia, photovoltaic (PV) capacity exceeded 41.8 GW by June 2025, surpassing peak load and typically driving energy surplus at midday with negative wholesale prices. However, as PV output declines and demand rises in the late afternoon, the system experiences sharp energy shortages and price spikes of several thousand dollars per MWh. This imbalance illustrates the characteristic 'duck curve,' in which midday oversupply is followed by steep evening ramping requirements. Electrification is also reshaping demand. In 2024, the uptake of electric vehicles (EVs) added an estimated 1.5 TWh to UK electricity consumption, while the installation of 4 million additional heat pumps raised peak demand by 18%. Charging and heating behaviours, shaped by diverse user preferences and daily routines, further amplify uncertainty in demand patterns.

Together, these factors reduce the predictability and controllability of energy systems, exposing the limitations of traditional approaches that historically relied on the flexibility of gas- or coal-fired plants and large fuel storage to absorb demand fluctuations and maintain system stability. In contrast, clean energy systems are shifting toward greater reliance on non-dispatchable renewables while progressively phasing out high-emission, dispatchable sources such as coal and oil. This trend reduces the inherent flexibility of conventional energy systems and amplifies the need for new, responsive solutions.

Flexibility refers to the capacity of energy systems to respond rapidly and effectively to short- and long-term variations in energy supply and demand while maintaining system stability, reliability, and efficiency. It spans multiple timescales, from sub-second grid frequency response to seasonal energy shifting; and operates across multiple energy vectors, including electricity, gas, heating, and hydrogen. Flexibility can be delivered through a broad portfolio of technological and systemic solutions: supply-side resources such as clean dispatchable generation, large-scale storage, and cross-border grid interconnectors; demand-side enablers including industrial load shifting, smart home energy management, and vehicle-to-grid (V2G) integration; and multi-vector integration such as power-to-gas and electrified heat. Increasingly, digital innovations such as real-time sensing, AI-based forecasting, and automated control are enabling responsive, decentralized flexibility, particularly from distributed energy resources (DERs). Yet flexibility is not purely a technical function: market design, regulatory frameworks, and social behaviours must also evolve to appropriately recognize, value, and incentivize flexibility services, particularly where these can substitute for or defer costly energy network reinforcements.

In this Review, we begin by identifying the flexibility needs of clean energy systems, followed by a comprehensive discussion of the technologies and strategies that enable their delivery. Our analysis is organized around three core domains: supply-side resources, demand-side enablers, and multi-vector integration. Within this framework, we assess the functionality, technical features, system-level contributions, and value propositions of each flexibility technology. We then evaluate global deployment trends, highlighting regional variations and technology-specific disparities, and give particular attention to the technical, economic, institutional, and social barriers that constrain their wider adoptions. Building on this analysis, we emphasize the systemic importance of flexibility and advocate a whole-system approach to accelerate clean energy transitions.

Flexibility needs [1203]

Fig. 1 | Flexibility needs in clean energy systems. Clean energy systems face increasingly flexibility needs due to the growing integration of variable renewable energy sources (vRES) such as wind and solar, the electrification of heat and transport via heat pumps and electric vehicle, and the decarbonization of gas infrastructure through green hydrogen. Panels (a-f) illustrate six key system challenges and the corresponding flexibility needs across the world. a, Power balancing, including long-duration balancing to manage multi-day renewable shortfalls (e.g., the UK Dunkelflaute event in December 2021) and short-term balancing to address day-ahead half-hourly forecast errors in wind generation. b, Load ramping to accommodate rapid changes in net load during late morning and early evening due to large-scale solar integration – illustrated by California Independent System Operator (CAISO) "duck curves" over three years. c, Congestion management and peak shaving to relieve grid bottlenecks and manage growing demand peaks associated with end-use electrification. d, Frequency response to stabilize system frequency following disturbances, with an emphasis on improving rate of change of frequency (RoCoF) and nadir in low-inertia, vRES-dominated systems. e, Voltage regulation to address localized overvoltage and undervoltage conditions, ensuring voltage quality and reliability across distribution networks. f, Resilience enhancement to support staged load restoration following the Spain-Portugal blackout. Panel a adapted from ref. x, NESO. Panel b adapted from ref. x, IEA. Panel c adapted from ref. x, xxx. Panel d adapted from ref. x, xxx. Panel e adapted from ref. x, xxx. Panel f adapted from ref. x, REE.

Energy system flexibility was traditionally delivered by adjusting supply to follow demand, with dispatchable generators able to ramp output up or down easily and quickly to maintain system balance and respond to contingencies. This approach relied on the controllability of fossil fuel plants, characterized by technical parameters such as ramp rates, start-up and shut-down times, and minimum up- and down-durations. Today, however, the energy transition is fundamentally reshaping both supply and demand, substantially increasing the need for flexibility. On the supply side, the rapid integration of vRES introduces greater short-term variability and forecast uncertainty. Simultaneously, these non-dispatchable sources are displacing conventional generators, diminishing access to traditional sources of flexibility. On the demand side, the electrification of heat and transport is introducing new, decentralized, and behaviour-driven loads that are more volatile and less predictable. Meanwhile, gas decarbonization and the growing role of hydrogen are deepening interdependencies across energy vectors, further complicating real-time operations. Together, these developments reduce system predictability and controllability, amplifying the need for enhanced flexibility across supply, demand, and multi-energy domains. Fig. 1 illustrates six representative flexibility needs in clean energy systems.

Power balancing maintains continuous real-time balance between electricity supply and demand, a fundamental operational requirement in clean energy systems. As generation becomes increasingly weather-dependent, flexibility needs arise across all timescales. In the short term (seconds to hours), flexibility is required to manage real-time deviations, such as those caused by sudden changes in weather affecting renewable output. Over longer timescales (days to seasons), flexibility addresses structural mismatches between when electricity is generated and when it is needed. A prominent example is the occurrence of prolonged vRES shortages, known as 'Dunkelflaute', which often coincide with high winter demand and place significant stress on generation adequacy. In December 2021, the UK experienced such an event, with daily average reliance on fossil fuels reaching 95% from 16-21 December to maintain system balance (Fig. 1a). This highlights the need for sufficient dispatchable capacity to reliably meet demand under variable and uncertain supply conditions.

Load ramping refers to the management of rapid changes in net electricity demand over short timeframes, primarily driven by fluctuations in vRES. In clean energy systems with high solar penetration, net load profiles often exhibit the 'duck curve' – marked by a midday dip caused by peak solar output coinciding with low demand, followed by a steep ramp in the early evening as solar declines and demand rises. This dynamic requires highly responsive flexibility over minutes to hours, particularly as traditional dispatchable ramping capacity is phased out, increasing residual load variability. The evolution of the California Independent System Operator (CAISO) duck curve exemplifies this challenge, with ramping needs exceeding 13 GW in just three hours (Fig. 1b). These trends highlight load ramping as a central flexibility need in renewable-dominated clean energy systems.

Congestion management ensures secure and efficient electricity flows within the physical and thermal limits of grid infrastructure. In clean energy systems, this challenge is exacerbated by the rapid deployment of DERs like rooftop solar, EVs, and heat pumps, as well as the geographic disconnect between renewable generation sites and load centers. These factors increase the risk of local or regional grid bottlenecks, particularly during periods of high renewable output or electrification-driven demand peaks. In early 2025, for example, grid congestion in the Netherlands left around 7,500 generation projects and 10,000 large users waiting to connect (Fig. 1c). Flexibility is needed to dynamically redirect generation, shift demand, or reconfigure grid flows in real time to alleviate such congestions without overloading line infrastructure. These timely, location-specific interventions, typically within minutes to hours, making congestion management a critical flexibility need in decentralized clean energy systems.

Frequency response is the continuous balancing of electricity supply and demand to maintain system frequency, typically at 50 or 60 Hz. Traditionally, frequency stability was inherently supported by the rotational inertia of large synchronous generators, which acted as physical buffers against sudden system disturbances. However, as clean energy systems increasingly rely on vRES such as wind and solar, much of this natural inertia is lost. This reduction leads to steeper and deeper frequency deviations – manifested as lower frequency nadirs and higher Rate of Change of Frequency (RoCoF) – following disturbances (Fig. 1d). Australia's National Electricity Market (NEM), for example, has experienced sharp frequency drops and rising RoCoF events due to declining inertia, prompting regulatory mandates for fast frequency response services. To maintain secure operation under these conditions, low-inertia clean energy systems require rapid and precise frequency response, typically within sub-second to second-level timescales.

Voltage regulation is essential for maintaining power quality and grid reliability, ensuring that voltage levels remain within acceptable operational limits across the power network. Traditionally, this function was provided by reactive power support from large synchronous generators. However, as clean energy systems shift toward vRES, this conventional source of voltage regulation is diminishing. Concurrently, the proliferation of DERs introduces bi-directional power flows and increased variability in local voltage profiles, creating localized overvoltage or undervoltage conditions (Fig. 1e) that existing grid infrastructure may struggle to manage effectively. In Malta, for example, high levels of rooftop solar have caused reverse power flows, leading to voltage rise and grid instability during oversupply periods. Clean energy systems thus require enhanced flexibility to provide dynamic, location-specific voltage regulation, with the ability to inject or absorb reactive power to stabilize voltage on seconds-to-minutes timescales.

Resilience enhancement during extreme events is an increasingly critical flexibility need. As energy systems shift from centralized, dispatchable generation to decentralized vRES, their capacity to withstand and recover from large-scale outages becomes more complex and less predictable. A recent example occurred on 28 April 2025, when a blackout in Spain disrupted electricity supply for 12 hours and caused a 15 GW load loss (Fig. 1f). Simultaneously, climate change is driving more frequent and severe extreme weather events, placing additional stress on energy system resilience. Historically, black start and restoration services were delivered by large synchronous generators, which offered high inertia and autonomous operation. With their progressive phase-out, clean energy systems must rely on a new portfolio of flexible resources capable of supporting load restoration under low-inertia or islanded conditions, with recovery timescales ranging from minutes to days. Ensuring their availability, coordination, and readiness will be essential to enhance resilience in a renewable-dominated, climate-vulnerable future.

Flexibility is not merely a technical attribute but a systemic enabler for reliable, cost-effective, and resilient clean energy systems. As fossil-based, centralized infrastructure is replaced by vRES, electrified end-use sectors, and DERs, the ability of the system to respond rapidly and efficiently to fluctuations becomes foundational. Without adequate flexibility, system operators risk increased renewable curtailment, underutilized assets, overinvestment in infrastructure, and reliance on costly backup peaking resources – all of which undermine the economic and environmental goals of energy transitions. Flexibility is also central to sector coupling and the cross-vector integration of electricity, gas, hydrogen, and heat, unlocking synergies that enhance efficiency and sustainability. In an increasingly uncertain energy landscape shaped by climate impacts, dynamic demand patterns, and evolving technologies, flexibility safeguards the system against unexpected fluctuations while enabling more proactive and adaptive planning for future energy needs. It must therefore be embedded not only as an ancillary capability, but also as a core principle guiding the design, regulation, and operation of clean energy systems.

Flexibility technologies [4062]

Fig. 2 | Flexibility technologies in clean energy systems. Flexibility is provided by a diverse portfolio of technologies spanning supply-side resources, demand-side enablers, and multi-vector integration. Panels (a-f) illustrate six representative examples. a, Clean dispatchable generation. Hydropower, biomass, flexible nuclear, and natural gas with carbon capture provide on-demand, low-carbon capacity to complement variable renewables. b, Grid interconnectors. High-voltage links enable cross-border power transfers, facilitating flexibility sharing and balancing of supply and demand over wide geographic areas. c, Large-scale electricity storage. Batteries and pumped hydro shift surplus renewable generation and release it during shortages. d, Long-duration energy storage. Hydrogen, linepack, and thermal storage allow seasonal shifting, for instance storing summer solar surplus for winter demand peak. e, Combined heat and power. Multi-vector integration of electricity, hydrogen and heat improves efficiency and delivers cross-sectoral flexibility through baseload and thermal demand. f, Demand-side response. Smart charging, flexible demand management, and vehicle-to-grid (V2G) help smooth evening peaks and complement renewable variability.

Fig. 3 | Characteristic, scale, and functionality of flexibility technologies. a, Spatial scale and typical size of fourteen flexibility technologies, ranging from domestic smart appliances and local EV charging to regional and national assets such as storage, interconnectors, and flexible generation. b, Response speed and operational duration of flexibility provision, showing complementarities between fast-acting, short-duration resources (for example, batteries, demand response) and slower but long-duration resources (for example, hydropower, power-to-X, gas networks). c, Contribution of each technology to six key flexibility needs. Solid circles denote the most promising providers, open circles indicate contributing roles, and dashes mark limited or irrelevant capability. The figure highlights that while individual technologies excel in specific services, no single option can cover all flexibility needs. A portfolio approach is therefore essential, combining short-term, mid-term, and long-duration flexibility across supply-side resources, demand-side enablers, and multi-vector integration.

Delivering flexibility in clean energy systems requires a diverse and complementary portfolio of technologies operating across multiple energy vectors, spatial scales (from household to international), and timescales (milliseconds to seasons). These technologies differ in response speed, operational duration, and maturity, yet together they address critical system flexibility needs. We group them into three domains: supply-side resources, demand-side enablers, and multi-vector integration (Fig. 2). For each domain, we assess their spatial and temporal characteristics alongside their functional contributions, highlighting how different technologies collectively underpin the reliable, efficient, and sustainable operation of clean energy systems (Fig. 3).

Supply-side flexibility technologies [1754]

Supply-side flexibility technologies enhance the controllability and responsiveness of clean energy systems by enabling real-time adjustments in generation or injecting stored energy during periods of peak demand or low renewable output. Key categories include clean dispatchable generation, large-scale electricity storage, and grid interconnectors.

Clean dispatchable generation [801]

In the pathway to net-zero emissions, solar and wind are projected to supply nearly 70% of global electricity by 2050. While increasingly cost-effective and technically viable at scale, the inherent intermittency of these weather-dependent resources poses significant challenges for real-time balancing. Clean dispatchable generation, capable of producing electricity on demand with minimal or zero greenhouse gas emissions, is therefore essential to bridging this gap. Key options include hydropower, clean natural gas, flexible nuclear, and biomass.

Hydropower is the largest global source of clean dispatchable generation, operating at regional-national scale with plants ranging from ~10 MW to over 10 GW. As a mature renewable technology, it offers rapid response capabilities, ramping within seconds to minutes, and operational durations spanning hours to months depending on reservoir size. Reservoir-based hydropower is particularly valuable for its ability to adjust generation quickly to match system demand, making it highly effective for short-term power balancing and load ramping. It also delivers essential ancillary services such as frequency response through synchronous inertia. Additionally, many hydropower plants are designated for black start, supporting system restoration during extreme events. Compared with run-of-river facilities, reservoir-based plants provide greater firm capacity by concentrating generation during peak periods. Recent advances in turbine design and digital control systems are further enhancing its ability to accommodate variability in net demand and manage congestion in geographically diverse grids. Collectively, these attributes make hydropower one of the most versatile and valuable technologies for supporting flexibility in clean energy systems.

Natural gas remains a globally important energy source owing to its storability, transportability (via pipelines or in liquefied form), and suitability for fast-ramping power generation. Modern clean natural gas plants typically operate at regional-national scale with capacities of 100 MW to over 1 GW, responding within minutes and sustaining output for hours to days. These characteristics make them well suited to provide both short-term (intra-day) and long-term (seasonal) flexibility. They are most promising for power balancing, load ramping, and resilience enhancement, while contributing to frequency response, voltage regulation, and congestion management. Gas-fired plants play a critical role in ensuring adequacy during periods of low vRES output or high demand. While cleaner than coal, natural gas still emits emitting substantial amounts of carbon dioxide (CO2) and methane, posing challenges for deep decarbonization. Carbon capture, utilization, and storage (CCUS) technologies can mitigate point-source emissions by up to 90%. A lower-carbon alternative is renewable natural gas (RNG), produced from biogas generated via anaerobic digestion or thermal gasification of organic waste. Once upgraded to biomethane, RNG can be injected into existing gas networks as a drop-in substitute, offering dispatchable, low-carbon flexibility without major infrastructure changes.

Flexible nuclear is evolving beyond its traditional role as a baseload generator to become a dynamic contributor to system flexibility. Advances in reactor design and control technologies now allow conventional nuclear plants to ramp output within hours, enabling partial participation in load-following operations. More notably, small modular reactors (SMRs), typically ranging from 50-300 MW per module and scalable to over 1 GW, are specifically designed for modular deployment and flexible operation. Operating at regional-national scale, they can adjust output within tens of minutes to several hours, with operational durations spanning daily cycling to continuous baseload generation. This flexibility enables nuclear power to accommodate variations in residual load and adjust output to follow changing demand patterns. It is thus most promising for supporting large-scale ramping requirements and enhancing system resilience, while also potentially contributing to power balancing, frequency response, and voltage regulation. Although still at the demonstration stage, SMRs are being piloted in China and Russia, showing strong potential as firm, low-carbon capacity. Their ability to integrate with cogeneration applications such as district heating or hydrogen production further extends their value in multi-vector systems, enhancing cross-sectoral flexibility and overall system efficiency.

Biomass-based generation is an inherently dispatchable resource that offers substantial operational flexibility, with plant sizes ranging from ~10 MW distributed engines to 500 MW centralized boilers, typically operating at local-regional-national scale. Start-up times range from minutes for fast-responding biogas engines to several hours for solid biomass boilers, enabling both rapid ancillary services and longer-duration output. Depending on fuel storage, operational durations can extend from sub-hourly frequency support to multi-day or even seasonal generation when dry biomass is stockpiled. As a mature and commercially available technology, flexible biomass provides firm generation capacity, meets thermal energy demand, and supplies grid services. It is considered most promising for supporting load-following requirements and enhancing system resilience, while also contributing to real-time power balancing, frequency response, and voltage regulation. Its system value depends critically on the sustainability of feedstocks and rigorous lifecycle emissions accounting to ensure genuine carbon reductions. When combined with thermal energy storage (TES) or integrated into district heating networks, biomass systems can strengthen local energy self-sufficiency and serve as a low-carbon substitute for fossil-based peaking plants.

Large-scale electricity storage [640]

Large-scale electricity storage plays a central role in delivering temporal flexibility across clean energy systems. By shifting energy over timescales ranging from seconds to days or even seasons, these technologies help absorb variability from vRES, smooth load ramps, enhance grid stability, and maintain power balancing under high shares of intermittent generation. The primary technologies deployed at scale include short-duration battery energy storage systems (BESS), and long-duration options such as pumped hydro storage (PHS), compressed air energy storage (CAES), and liquid air energy storage (LAES).

BESS, predominantly based on lithium-ion chemistry, offer ultra-fast response capabilities, with discharge initiation occurring within milliseconds to seconds and typical operational durations of 1-6 hours. At scales ranging from household units in the kW range to utility projects exceeding 100 MW, they provide flexibility across domestic to national levels. With round-trip efficiencies of 85-95%, BESS are particularly effective for short-term frequency response in low-inertia systems, where even small imbalances can cause rapid deviations. They are also most promising for voltage regulation and real-time power balancing, while also contributing to load shifting, congestion management, and emergency resilience. When co-located with solar and wind farms, they help firm renewable output, reduce curtailment, and enhance system adequacy. As the technology matures, next-generation chemistries such as sodium-ion, solid-state, and flow batteries are emerging to improve safety, sustainability, and scalability for longer-duration applications, extending BESS capabilities into multi-day storage and broadening their role in clean energy systems.

PHS is the most mature form of large-scale electricity storage and remains a backbone of long-duration and system-wide flexibility worldwide. By using surplus electricity to pump water from a lower to an upper reservoir, PHS stores energy as gravitational potential that can later be released back to electricity through hydraulic turbines when needed. PHS typically operates at national scale, with unit capacities ranging from 100 MW to over 3 GW. They deliver sub-minute response and discharge durations of 6-20+ hours, extending to multi-day or even seasonal operation where reservoirs are large enough. Round-trip efficiencies average 70-85%, and operational lifespans often exceed four decades, making PHS a durable and cost-effective asset. It is most promising for daily and weekly power balancing, load ramping, and resilience enhancement, while also contributing to frequency response and voltage regulation through synchronous generation and inertia provision. Importantly, PHS also deliver seasonal flexibility by storing water in large reservoirs over extended periods, helping to mitigate structural mismatches between supply and demand, particularly during prolonged renewable shortfalls, such as Dunkelflaute events. With its high capacity, long discharge duration, and proven ability to stabilize grid across multiple timescales, PHS serves as a cornerstone for system adequacy and reliability in clean energy systems.

CAES and LAES are emerging long-duration flexibility technologies based on mechanical energy conversion. CAES stores electricity by compressing air into underground caverns or aboveground tanks, later releasing it to drive turbines when needed. Projects typically operate at regional-national scale with unit capacities of 50-300 MW, response times of several minutes, and discharge durations of 8-24 hours, making them well suited for daily load shifting, intra-day power balancing, and backup supply during renewable shortfalls such as Dunkelflaute events. LAES, by contrast, liquefies ambient air using surplus renewable electricity and stores it in insulated cryogenic tanks for later regasification and expansion through turbines. Current projects are smaller, at 5-50 MW, but deliver sub-10-minute response and 6-20+ hours of storage, offering adaptable support for peak demand, resilience, and local congestion management. With modular designs and compatibility with urban or industrial sites, LAES enables geographically distributed flexibility, complementing the bulk scale of CAES. Together, these technologies are most promising for load ramping, power balancing, and resilience enhancement, while also contributing modestly to frequency response, voltage regulation, and congestion relief. By extending the duration and scope of storage services, CAES and LAES enhance renewable integration and strengthen system reliability across multiple timescales.

Grid interconnectors [267]

Grid interconnectors are high-capacity transmission links, typically utilizing high-voltage alternating current (HVAC) or high-voltage direct current (HVDC) technologies, that connect electricity grids across regions or national boundaries. Operating at national-international scale, they typically range from 100 MW to 2 GW per link, with response times of seconds to minutes and effectively continuous duration of operation. Interconnectors provide system-level flexibility by enabling real-time power transfers across geographic areas, facilitating resource sharing, supporting cross-border power balancing and integrating large shares of vRES. Their capacity for spatial balancing is especially valuable when generation and demand are geographically decoupled – for example, transferring surplus wind from coastal regions or solar power from high-insolation zones to distant demand centers experiencing supply deficits.

As a technologically mature solution with decades of deployment in Europe, North America, and the Nordic countries, interconnectors have consistently demonstrated their ability to reduce renewable energy curtailment, displace fossil peaking plants, and enhance resource adequacy through mutual reserve sharing – without requiring each region to maintain redundant backup capacity. This shared flexibility supports cost-effective system planning and helps defer investments in domestic generation or transmission infrastructure. Specifically, interconnectors are most promising for congestion management, power balancing, and resilience enhancement, while also contributing to load ramping, frequency response, and voltage regulation depending on system conditions. Modern advances such as Dynamic Line Rating (DLR) and Flexible AC Transmission Systems (FACTS) further enhance network controllability, enabling interconnectors to adapt power flows dynamically and expand their role in system stability. Collectively, they are a mature and proven enabler of both spatial and temporal flexibility, facilitating cross-border renewable integration and strengthening multi-national energy system resilience.

Demand-side flexibility technologies [1113]

The accelerating digitalization and electrification of end-use sectors are transforming consumers from passive energy users into active providers of system flexibility. Demand-side flexibility, typically enabled through structured demand-side response (DSR) programs, has emerged as a scalable, distributed, and cost-effective complement to traditional supply-side flexibility. By dynamically adjusting consumption in real time or strategically shifting demand across hours or days, DSR allows electricity use to respond to system conditions, market signals, or direct control commands from grid operators or aggregators. This enables residential, commercial, industrial, and transport sectors to contribute to grid flexibility, reducing the need for additional generation or costly network reinforcements.

Domestic smart appliances [319]

Smart thermostats, washing machines, refrigerators, water heaters, and HVAC (heating, ventilation, and air conditioning) systems are emerging as key enablers of residential demand-side flexibility. When integrated with smart meters and home energy management systems (HEMS), these internet-connected devices can automatically adjust their operations in real time or follow user-defined preferences in response to price signals, grid conditions, or direct control commands. Operating at household scale, individual devices typically range from 0.1-5 kW, but when aggregated across millions of households they represent portfolios in the GW range. With fast response speeds from sub-seconds to minutes and operational durations of tens of minutes to several hours, smart appliances offer a decentralized and adaptable source of flexibility.

They are most promising for fast frequency response and short-term power balancing, while also contributing to congestion management, voltage regulation, and the smoothing of rapid demand ramps. In practice, coordinated appliance control shifts household electricity use away from peak periods and toward times of surplus renewable output, such as midday solar generation. This demand modulation flattens load profiles, reduces renewable energy curtailment, and improves grid efficiency. In low-inertia systems, their rapid adjustability helps stabilize frequency, while voltage sensitivity enables localized support for distribution networks. During congestion events, aggregated control can ease stress on feeders, and in emergencies staged restoration can be facilitated by curtailing non-critical loads and safeguarding essential services.

While time-of-use (ToU) tariffs enabled by smart meters provide a foundation for participation, broader engagement will likely depend on more dynamic pricing structures, such as real-time or critical-peak tariffs, as well as improved access to local flexibility markets. Scaling up will also require simplified participation frameworks, greater automation, and robust cybersecurity. As smart meter penetration increases and market access expands, domestic smart appliances are well positioned to become a cornerstone of distributed flexibility. By delivering fast, responsive, and consumer-friendly demand modulation, they can support a wide range of flexibility needs in clean energy systems.

Industrial and commercial demand response [376]

Industrial and commercial demand response (ICDR) refers to the adjustment of electricity consumption by non-residential energy users – such as manufacturers, commercial buildings, public sector facilities, and increasingly data centers – in response to price signals, grid conditions, or contractual obligations. With high electricity intensity and large controllable loads, relatively few participants can deliver substantial flexibility.

Operating at local and regional scales, ICDR portfolios typically range from 100 kW to over 100 MW. Response speeds typically range from seconds to minutes for ancillary services such as frequency response, and hours for load shifting. Operational durations may involve short-term adjustments from minutes to hours or sustained shifts over full-day or even weekly cycles, depending on the underlying process and operational constraints. Flexibility is often embedded in industrial activities such as electrolysis, aluminium smelting, and cold storage, as well as in commercial building systems such as HVAC, lighting, and refrigeration. With the rapid growth of AI and digital services, data centers are also emerging as a major source of ICDR flexibility, given their large, controllable loads, and potential to shift non-time-critical computing tasks in response to system needs.

By enabling demand to follow supply, ICDR contributes to multiple flexibility needs in clean energy systems. It supports real-time power balancing by modulating large-scale loads, reduces ramping stress by smoothing transitions between low and high net load, and provides frequency response through automated, rapid-response controls. ICDR also plays a role in capacity adequacy by participating in peak shaving and capacity markets, especially during seasonal demand peaks or extreme weather events. Furthermore, by providing geographically targeted demand reductions, ICDR helps relieve network congestion and can defer the need for distribution grid upgrades. In addition, it can contribute to voltage regulation through localized load adjustments and enhance short-term resilience by reducing demand during system disturbances, though these remain secondary benefits compared to its core services.

Participation in ICDR programs depends on factors such as operational feasibility, automation readiness, and market incentives, supported by advanced metering and real-time monitoring infrastructure to ensure reliability without disrupting core operations. When properly integrated into market mechanisms, ICDR offers a scalable, cost-effective, and highly reliable form of demand-side flexibility that complements supply-side resources and strengthens the efficiency, resilience, and decarbonization of clean energy systems.

Smart and bidirectional charging [310]

Electric vehicles (EVs) spend approximately 95% of their time parked, creating a significant yet underutilized opportunity to provide system flexibility. With EV adoption accelerating, their integration through smart charging (V1G) and bidirectional charging technologies – collectively known as vehicle-to-everything (V2X) – is transforming them from passive loads into active, distributed storage resources at domestic-local scale.

Smart V1G is the most commercially mature solution. It enables unidirectional charging flow from the grid to the vehicle, with schedules dynamically optimized in response to electricity prices, user preferences, grid constraints, or renewable availability. Typically operating over short-to-medium durations of 2-8 hours and responding within minutes, V1G allows charging demand to shift away from peak periods and toward times of surplus renewable output, thereby easing congestion on local distribution networks and reducing system stress without compromising user mobility.

Bidirectional V2X extends these capabilities by allowing EVs to discharge electricity back to the grid or local loads. In vehicle-to-grid (V2G) mode, fleets act as mobile storage units, providing frequency response, voltage regulation, peak shaving, capacity adequacy, and load-following services. Vehicle-to-home (V2H) and vehicle-to-building (V2B) configurations further enhance resilience by supplying behind-the-meter power during price spikes or renewable shortfalls, improving energy self-sufficiency. At the device level, chargers typically range from 3-22 kW for AC and 50-350 kW for DC fast charging, but aggregated fleets can provide flexibility at the 100s of MW scale.

With response times of seconds to minutes and operational durations from minutes to several hours depending on battery size, state of charge, and dwell time, EV smart and bidirectional charging is most promising for frequency response, real-time power balancing, and congestion management, while also contributing to voltage regulation, smoothing of short ramps, and short-term resilience. Supported by advancing digital infrastructure, evolving standards, and new business models, EVs are increasingly recognized as ''storage on wheels'' – a fast-response, scalable flexibility resource that strengthens clean energy transitions.

Multi-energy flexibility technologies [1099]

Multi-energy flexibility technologies – also referred to as cross-vector or sector-coupling solutions – enable the coordinated conversion, storage, and exchange of energy across multiple carriers, including electricity, gas, hydrogen, and heat. These technologies enhance flexibility by allowing surplus energy in one vector to be shifted, transformed, or stored to meet demand in another, thereby improving overall efficiency, reliability, and resilience. At their core, multi-energy systems (MES) are designed to manage different energy vectors in an integrated manner through shared infrastructure and coordinated control, exploiting intersectoral synergies to optimize resource utilization. Key MES flexibility technologies include power-to-X (P2X) conversions, gas networks with linepack storage, and district heating with thermal energy storage (TES), which together coordinate cross-vector energy flows at community, regional, or national scales.

Power-to-X conversion [423]

P2X technologies provide vital MES flexibility by converting surplus electricity, typically from vRES, into alternative energy carriers such as gas, heat, hydrogen, or synthetic fuels. In doing so, they not only decarbonize hard-to-abate sectors like transport, industry, and heating, but also support grid balancing, long-duration energy storage, and cross-sector resilience. Operating mainly at regional to national scale, today's projects range from 1-100+ MW, with GW-scale facilities planned. Response times are typically minutes to hours, while storage durations extend from days to entire seasons depending on the carrier.

Among the most mature options is power-to-gas (P2G), which uses electrolysis to produce green hydrogen from electricity and water. The resulting hydrogen can be stored for durations from hours to seasons, injected into gas networks or used directly in industrial processes and fuel cells. It can also be synthesized into renewable fuels like methane or ammonia, improving dispatchability and transportability. P2G provides medium- to long-duration storage and helps address structural supply-demand mismatches, including prolonged renewable shortfalls such as Dunkelflaute events. It also contributes to power balancing by modulating electrolysis operation in response to real-time grid conditions and enables cross-sectoral flexibility by linking electricity and gas systems.

Power-to-heat (P2H) technologies, including resistance heaters, heat pumps, and electrode boilers, convert electricity into thermal energy for direct use or storage in thermal buffers like hot-water tanks or district heating systems. With fast response speeds of seconds to minutes and short to medium storage durations from hours to days, P2H provides agile demand-side flexibility, supports voltage regulation, helps manage peak loads, and absorbs renewable fluctuations locally. When paired with TES, P2H can decouple electricity use from heat demand, easing stress on both distribution networks and upstream generation assets.

Power-to-liquids (P2L) combines renewable electricity with captured CO2 to produce synthetic fuels such as methanol or e-kerosene. These high-energy-density liquids offer long-duration storage and are compatible with existing transport infrastructure, making them particularly important for decarbonizing aviation, maritime transport, and heavy industry. Although less mature and more costly than P2G or P2H, P2L expands the role of renewable electricity in sectors where direct electrification is impractical. By storing renewable energy in liquid form, P2L provides seasonal flexibility and capacity adequacy during periods of low renewable availability.

Taken together, P2X technologies are most promising for load ramping and resilience enhancement, and contribute to power balancing and congestion relief, though they are not suited for frequency or voltage services. Their strategic deployment will be critical for adapting clean energy systems to high renewable penetration, accelerating electrification, and managing growing interdependencies across infrastructures.

Gas networks with linepack storage [261]

Gas networks, particularly using linepack storage, represent a valuable source of MES flexibility by exploiting the compressibility of gas within transmission pipelines. By modulating pipeline pressure, network operators can temporarily store or release significant volumes of gas, providing intra-day and short-term power balancing without requiring additional infrastructure. Operating at regional to national scale, gas networks offer effective deliverability ranging from hundreds of megawatts to several gigawatts. Response times are typically minutes to hours, with operational durations spanning days to weeks, and extended further when integrated with centralized gas storage in salt caverns, aquifers, depleted reservoirs, or liquefied natural gas (LNG) terminals. This enables the gas system to support both base and peak loads and bolster adequacy during extreme weather events or Dunkelflaute events when renewable supply is constrained.

As gas grids decarbonize through the introduction of green hydrogen and RNG, linepack is expected to play a growing role in transporting and storing low-carbon gases, facilitating sector coupling between electricity and gas. Specifically, it can absorb and store excess renewable electricity indirectly via P2G and redistribute this energy through the gas network as needed. With high technical maturity, embedded system availability, and low marginal cost, gas networks are most promising for ramping and resilience services, while also contributing to power balancing and congestion relief, though they play little role in frequency response or voltage regulation. By smoothing mismatches between intermittent renewables and dynamic demand, linepack provides an efficient, cost-effective buffering mechanism that complements other flexibility options and enhances the resilience, affordability, and reliability of integrated clean energy systems.

District heating with thermal energy storage [290]

District heating systems, when integrated with TES, provide a highly scalable and cost-effective form of flexibility by decoupling heat production from consumption. These systems distribute thermal energy from low-carbon sources such as large-scale heat pumps, geothermal systems, or industrial waste heat through insulated pipelines to supply residential, commercial, or industrial users. When coupled with TES units like stratified hot water tanks or molten salt reservoirs, district heating can absorb surplus renewable electricity through P2H technologies during periods of low demand or high vRES output, and store the heat for later use.

Operating at local to regional scale, systems typically range from 1-500 MWth, with seasonal TES extending into the GWth range. Response times are minutes to hours, with operational durations spanning hours to multiple days, and even seasonal in some configurations. District heating with TES is most promising for ramping and resilience services, while also contributing to power balancing and congestion relief, though it does not provide frequency or voltage control. This flexibility is particularly valuable in dense urban and industrial settings where electrification of heat is otherwise complex and costly. TES enables short-term balancing and peak shaving by shifting heating demand to off-peak periods, while longer storage durations enhance system adequacy during renewable shortfalls.

Many networks also integrate hydrogen-fueled combined heat and power (CHP) plants, which can adjust heat-to-power ratios in response to grid signals. Used alongside TES, CHP units can run during off-peak electricity periods, store thermal energy, and dispatch later, thereby supporting renewable load-following and relieving network congestion. Technologically mature, with long operational lifespans and low thermal losses, district heating with TES strengthens electricity-heat sector coupling, promotes local self-sufficiency, and represents a foundational pillar of integrated, resilient, and decarbonized energy systems.

Flexibility strategies [1721]

Fig. 4 | Global deployment of flexible technologies. Panels (a-h) show the geographic distribution and leading countries for eleven key flexibility technologies. a, Clean dispatchable generation. Hydropower, biomass, carbon capture and storage (CCS), small modular reactors (SMR) and hydrogen projects, with deployment concentrated in Asia, the Americas and Europe. b, Energy storage. Mechanical, thermal, electrochemical and chemical storage, dominated by pumped hydro and batteries in East Asia, the United States and Europe. c, Gas infrastructure. Import and export terminals, with Europe as the largest importer and North America, Russia, the Middle East and Australia as leading exporters. d, Grid interconnectors. Cross-border electricity interconnections, most extensive in Europe, with additional deployments in the Americas and Asia. e, Smart appliances. Household and commercial devices with demand response capabilities, concentrated in high-population regions. f, Data centres. Digital infrastructure with flexible load potential, clustered in North America, Western Europe, East Asia and Australia. g, Charging stations. Electric vehicle charging infrastructure, expanding rapidly in China, North America and Western Europe. h, National leaders. Top three countries by deployment across the eleven flexibility technologies, with China, the United States and European nations leading, alongside Brazil, Australia and Canada in specific domains.

Table 1 | Barriers to broader deployment of flexibility technologies

| Barrier type | Description | Affected technologies |
| --- | --- | --- |
| Technical | Limited technological maturity, long lead times, integration complexity, and lack of interoperability across energy vectors. | Long-duration storage, CCS, SMRs; sector coupling of power-to-gas and district heating. |
| Economic | Flexibility services undervalued in markets; uncertain revenue streams; high upfront capital costs; cost gaps with incumbent options. | Grid-scale storage, demand response, interconnectors, CCS, SMRs. |
| Regulatory / Institutional | Outdated grid codes, fragmented governance, slow permitting and siting, uncertain safety and certification standards for new fuels. | Hydropower, pumped hydro, transmission lines, hydrogen and gas networks. |
| Social / Behavioural | Low consumer awareness and participation; privacy and data concerns; opposition to large infrastructure due to environmental or safety risks. | Smart appliances, dynamic tariffs, CCS projects, nuclear plants, transmission corridors. |

Box 1 | Flexibility technology policy in the United Kingdon

Fig. 5 | Whole-system energy approach – facilitate flexibility technology toward clean energy transition.

Global deployment [606]

Flexibility technologies are being deployed at increasing scale across global energy systems, though with significant regional variation in adoption rates, technology preferences, and policy priorities (Fig. 4). The pace of deployment is largely shaped by each country's decarbonization strategies, renewable penetration levels, and regulatory frameworks.

Supply-side flexibility. Clean dispatchable generation (Fig. 4a) – including hydropower, biomass, CCS, SMR, and hydrogen production – remains unevenly distributed. Hydropower dominates in Asia, Europe, and North America, while biomass is concentrated in Europe and South America. CCS projects are largely confined to North America and parts of Europe, whereas SMRs are currently demonstrated only in China and Russia. Hydrogen facilities are dispersed globally but remain at pilot scale outside a few leading countries, such as Germany, the United States (US), and France.

Storage technologies. Large-scale storage capacity (Fig. 4b) is concentrated in China, the US, and Europe. Mechanical storage, led by pumped hydro, dominates in mountainous regions, while electrochemical storage is scaling rapidly in the US, China, and several European countries. Emerging thermal and chemical storage projects remain modest in scale but are beginning to expand in specialized applications.

Gas networks. Gas import and export infrastructure (Fig. 4c) underscores strong interdependencies across regions, particularly in Europe and Asia, where security of supply is a central driver of such flexibility investments. These projects are predominantly coastal, reflecting reliance on seaborne LNG trade. Export thermals are concentrated in North America, Russia, the Middle East, and Australia, while Europe and East Asia are the largest importers.

Grid infrastructure. Interconnectors (Fig. 4d) are most extensive in Europe, where cross-border links underpin regional electricity market integration. North and South America and parts of Asia also host major interconnections, though deployment is less dense. In North America, for example, the US is a net electricity importer from Canada, with high-capacity interconnections enabling Canadian hydropower exports to support US demand.

Demand-side flexibility. Smart appliances (Fig. 4e) and data centres (Fig. 4f) are emerging as distributed, responsive demand resources. Deployment is concentrated in high-population, high-income regions – notably North America, Europe, and East Asia (for example, China and Japan) – where digital infrastructure and consumer adoption are advanced. Data centres, which represent both large flexible loads and potential providers of demand response, are clustered around major population and technology hubs such as the US.

Transport flexibility. EV Charging infrastructure (Fig. 4g) is expanding rapidly, led by the US, Europe and China. Deployment density, however, varies widely, with large-scale rollouts concentrated in advanced economies to support accelerating EV adoption.

Cross-country leaders. Deployment rankings (Fig. 4h) highlight national leadership in flexibility portfolios. China, the US, and European countries consistently rank among the top three across the most of flexibility technologies, reflecting strong policy support, large energy demand, and advanced industrial capacity. Other notable leaders include Brazil in hydropower and biomass; Australia and Canada in CCS; Russia in SMRs; Japan in storage, gas networks, and EV charging infrastructure; and Thailand in grid interconnectors.

Taken together, these patterns reveal both progress and imbalance. Large-scale deployment of flexibility technologies – such as clean dispatchable generation, grid-scale storage, and interconnectors – has so far been concentrated in regions with advanced energy systems and mature energy markets, while distributed and renewable-linked solutions, including smart appliances, demand response, and EV charging, are beginning to scale more widely across diverse regions worldwide. Storage and demand-side enablers are expanding fastest, while CCS, SMRs, and hydrogen remain at early stages. Regional interconnectors and LNG trade highlight continued reliance on cross-border cooperation for resilience enhancement of energy systems. Achieving net-zero targets will require not only accelerated deployment across all flexibility domains but also broader geographic diffusion to ensure equitable and resilient global energy transitions.

Barriers to broader deployment [532]

Despite accelerating deployment, multiple barriers continue to constrain the wider adoption of flexibility technologies. These challenges are multidimensional – technical, economic, institutional, and social – and vary by technology, region, and stage of market development.

Technical barriers remain substantial. Many flexibility technologies, such as CCS, SMRs, and long-duration storage, are still at early stages of commercial readiness. High capital intensity, long lead times, and demonstration-scale maturity limit scalability. Even mature technologies like batteries and pumped hydro face challenges of resource availability, site constraints, and system integration complexity. Interoperability across energy vectors (electricity, gas, hydrogen and heat) is also limited, complicating sector coupling and the coordination of multi-vector flexibility. For instance, the UK's Project Union seeks to repurpose existing natural gas networks for hydrogen delivery and storage, yet differences in material compatibility, safety standards, and gas quality regulations present significant obstacles.

Economic barriers stem largely from inadequate market design. Flexibility services are often undervalued, as most markets reward energy provision rather than fast response, locational benefits, or energy resilience. This creates revenue uncertainty for investors in technologies such as storage, demand response, and interconnectors. Increasing electricity price volatility adds further risk. In Germany, for instance, high renewable penetration has frequently driven wholesale electricity prices negative. Under current energy-only market designs, such volatility discourages investment in clean dispatchable generation, since producers face uncertain or even negative returns during periods of oversupply. Cost competitiveness also varies widely: while short-duration batteries have seen rapid cost declines, long-duration storage, CCS, and SMRs remain expensive prohibitively expensive without supportive finance or policy mechanisms.

Regulatory and institutional barriers slow deployment across multiple fronts. In many regions, fragmented governance divides responsibility for electricity, gas, and heat systems across separate agencies or regulatory authorities, complicating coordinated planning and investment in flexibility technologies. This is evident in the US, where overlapping authority between federal, state and regional bodies makes integrated decision-making particularly difficult. Outdated grid codes further constrain participation by failing to recognize the technical capabilities of modern assets such as batteries and demand response. In the UK, for example, the absence of co-optimization between energy and frequency response markets prevents aggregated flexibility resources from contributing frequency and inertia services to its high-renewable, low-inertia power system. Permitting and siting procedures represent another major bottleneck. Large infrastructure projects – particularly hydropower, pumped hydro, and cross-border transmission lines – require complex environmental assessments, land-use approvals, and stakeholder consultations, processes that are often lengthy and politically contested, delaying or even preventing deployment. For example, Germany's SuedLink transmission project was delayed for years due to local opposition to overhead lines, while concerns over national security blocked China's involvement in UK new nuclear power projects.

Social and behavioural barriers add further complexity. Consumer engagement in demand-side flexibility is limited by low awareness, privacy concerns, and the perceived complexity of smart appliances or dynamic tariffs. Even where enabling technologies are available, many households and businesses are reluctant to shift consumption patterns without clear incentives or user-friendly platforms. In the UK, uptake of smart meters and time-of-use tariffs has lagged expectations. Trials such as Octopus Energy's Agile Tariff show that EV-owning households reduced evening peak demand by nearly 50%, yet most other households exhibited only modest shifts in consumption.

Whole-system approach [546]

Overcoming these barriers requires moving beyond incremental, technology-by-technology solutions toward a whole-system approach. This perspective emphasizes coordination across energy vectors, timescales, geographies, digital platforms, markets, and social systems, ensuring that diverse resources operate in concert to deliver a reliable, affordable, and sustainable energy future.

Multi-vector integration is a central pillar. Electricity, gas, heat, and hydrogen systems are often planned and operated in isolation, yet their interdependencies can be harnessed to provide flexibility across multiple energy domains. P2G technologies, for example, enable surplus renewable electricity to be converted into hydrogen for storage and later reconversion, while district heating networks equipped with TES can absorb excess renewable output and reduce demand peaks in the power system. Gas linepack and LNG infrastructure, when aligned with hydrogen blending and storage strategies, extend balancing capabilities to seasonal scales, enhancing overall system adequacy.

Flexibility must also be coordinated across temporal and spatial dimensions. Needs range from sub-second frequency response to seasonal balancing, and no single technology can address this full spectrum. Batteries and demand response provide rapid, short-duration services, while reservoir hydropower, pumped hydro, and hydrogen storage deliver longer-term adequacy. Spatially, cross-border grid interconnectors expand the pool of flexible resources, reducing local mismatches between generation and demand. Coordinating deployment across timescales and geographies helps aviod overinvestment in single solutions and unlocks synergies between assets.

Digitalization and orchestration are critical enablers. Advances in AI-driven forecasting, smart metering, and automated control platforms enable system operators to manage millions of DERs – from EVs and heat pumps to smart appliances and data centres – alongside large-scale assets. Virtual power plants and demand aggregators can pool small, flexible loads into competitive portfolios that provide system services. Digital infrastructure thus acts as the connective tissue that transforms fragmented assets into a coherent flexibility system.

Delivering a whole-system approach also depends on market and regulatory alignment. Current energy-only market designs rarely provide sufficient revenue certainty for flexibility investments. Instead, flexibility must be explicitly valued through co-optimized energy, capacity, and ancillary service markets. Updated grid codes should recognize the technical capabilities of inverter-based resources and aggregated demand, while streamlined permitting processes are needed to accelerate large-scale projects such as interconnectors and storage facilities. Cross-border market integration further enhances flexibility by enabling regions to share resources, as seen in the EU's internal electricity market.

Finally, a whole-system strategy must also incorporate socio-technical integration and governance reform. Public engagement and consumer participation are essential to unlocking demand-side flexibility. Policies must ensure that benefits are equitably distributed, rather than concentrated among households with EVs or rooftop solar. Transparent communication, community ownership, and fair cost-benefit sharing can strengthen social legitimacy and build trust in flexibility-driven energy transitions. On the institutional side, integrated long-term planning and governance frameworks are essential to align infrastructure investment, digital innovation, and distributed resource deployment. Emerging initiatives, such as the UK's Future System Operator and the EU's sector-coupling strategies, illustrate steps toward more coordinated system governance.

In sum, a whole-system approach treats flexibility not as a set of isolated technologies, but as an integrated capability that spans technical, economic, and social dimensions. By orchestrating supply-side, demand-side, and multi-vector resources within supportive markets and governance structures, clean energy systems can maximize flexibility and ensure secure, affordable, and low-carbon transitions.

Summary and future perspectives [300]

Together, these barriers highlight the need for coordinated strategies that combine technological innovation, supportive market frameworks and effective governance. Overcoming them will be essential to unlock investment, accelerate deployment and ensure flexibility technologies deliver their full potential in supporting secure, affordable and decarbonized energy systems.

the UK established the National Energy System Operator (NESO) for electricity and gas in October 2024, taking over responsibilities from the National Grid's Electricity System Operator (ESO), Enabled by the 2023 Energy Act, the publicly owned, operationally independent NESO is responsible for strategic, long-term planning and real-time management of both systems to support the UK's transition to net zero emissions by 2030.

Countries with a sustainable consumption and production national action plan: This metric measures whether countries have implemented policies as part of a sustainable consumption and production plan. This does not measure the quality or ambition of these plans, only whether a country has one: https://ourworldindata.org/grapher/country-scp-plan

When will countries phase out coal power? This measures pledges to phase out coal from the electricity mix: https://ourworldindata.org/grapher/coal-phase-out-timeline

Aggregators and digital platforms [147]

To enable broad participation in flexibility markets across sectors, aggregators and digital platforms are emerging as key intermediaries that coordinate and manage distributed energy resources (DERs) such as residential batteries, EVs, heat pumps, and smart appliances. By pooling these decentralized assets, aggregators form virtual power plants (VPPs) capable of delivering fast-response (seconds to minutes), short- to medium-duration flexibility across multiple sectors.

These platforms use real-time data, AI-driven algorithms, and standardized communication protocols to optimize DER dispatch in response to market prices, grid conditions, or control signals. VPPs can participate in wholesale markets, balancing services, and local flexibility schemes – offering grid operators a scalable and decentralized alternative to conventional peaking plants.

While still evolving, the technological maturity of aggregator platforms is rapidly advancing. Their deployment enhances grid stability, improves renewable integration, and unlocks new revenue opportunities for consumers – supporting a more resilient, efficient, and participatory clean energy system.

Energy hubs and integrated energy systems [165]

Energy hubs are localized, multi-vector infrastructures – often located in campuses, communities, or industrial parks – that manage the production, conversion, storage, and consumption of multiple energy carriers, including electricity, gas, and heat. By leveraging real-time data, digital controls, and coordinated storage assets, these hubs optimize local energy flows, enable demand-side flexibility, and facilitate the integration of variable renewable energy at the distribution level. Their ability to shift loads across vectors and time enhances system reliability, reduces peak demand, and supports decarbonization in decentralized energy settings.

At broader scales, integrated energy systems operate across regional or national boundaries, coordinating cross-sectoral energy flows using advanced forecasting, automated dispatch, and market-based mechanisms. These systems enable cross-vector flexibility – shifting energy in time (short- to long-term), form (electricity, thermal, gas), and space (local to national) – to absorb variability, maintain system balance, and ensure efficient use of resources. As such, they form the backbone of a resilient, interoperable, and sustainable clean energy future, supporting deep decarbonization across power, heat, transport, and industry.

Investing in flexibility is a no-regrets decision as it delivers material net savings of up to £16.7bn/yr across all net zero scenarios analysed in 2050

Flexibility supports a net zero energy system to cope with dark, cold and windless days in winter

Embedding flexibility in zero carbon heat and transport solutions will help to reduce their system impact and costs making their decarbonisation economically more feasible

Flexibility is deployed more locally in 2050 and delivers significant value nationally 

Reaching net zero by 2050 whilst meeting security of supply requires unprecedented build-out across the energy system

Pushing the energy system to go beyond zero carbon has material cost and infrastructure implications

Carbon negative technologies have an important role in helping to meet the net zero target in 2050

Any single heating solution dominant future has a significant cost and infrastructure impact on the energy system in 2050

The use of hydrogen across the energy system brings carbon and cost benefits and requires a portfolio of production methods and availability of CCS infrastructure

Flexibility should be integrated into enabling infrastructure including low carbon heat and transport solutions from the start

Consumer engagement on flexibility beyond just commercial value is a critical aspect to scaling up flexibility technology deployment

An evolving regulatory environment, combined with potentially low financial gains in the long term, creates challenges for business model development

A smart and flexible system can only be enabled by digitalisation of the energy system

Continued efforts for new technology development and innovation focused on cross vector integration is important to have them ready in time