Flexibility in clean energy systems

Dawei Qiu1,2, Danny Pudjianto1, & Goran Strbac1

1Department of Electrical and Electronic Engineering, Imperial College London, UK. 2Department of Engineering, University of Exeter, UK.

Abstract [203]

As global energy systems transition toward net-zero emissions, maintaining real-time balance between supply and demand – known as energy system flexibility – has become a critical challenge. This challenge arises from the increasing penetration of variable renewable energy, the electrification of heat and transport, and the decarbonization of gas infrastructure, all of which introduce unprecedented variability and operational uncertainty. In this Review, we examine the technologies and strategies that provide flexibility across clean energy systems, enabling reliable, efficient, and low-carbon energy delivery. We categorize flexibility into electricity system flexibility and gas and hydrogen system flexibility, encompassing both supply-side and demand-side mechanisms. Technologies such as clean dispatchable generation, storage, interconnectors contribute to electricity system flexibility, while linepack, hydrogen electrolysis, and gas-fired generation offer critical balancing services in gas and hydrogen networks. Simultaneously, demand-side response – enabled by digitalization and decentralization – is emerging as a scalable, distributed source of flexibility across the residential, commercial, industrial, and transport sectors. We also discuss key barriers to the widespread deployment of flexibility technologies and explore how market structures and regulatory frameworks must evolve to unlock investment in flexibility at scale. Ultimately, achieving a decarbonized energy future will require the coordinated integration of diverse flexibility technologies across interconnected multiple energy vectors.

Introduction [650]

Global efforts to decarbonize energy systems are accelerating in response to the urgent need to mitigate climate change. Central to this transformation is the widespread electrification of end-use sectors such as transport and heating, alongside the rapid deployment of low-carbon energy sources. These include variable renewable energy (VRE) sources such as wind and solar, and emerging green fuels like hydrogen. [global installed capacity of solar and wind in GW, % share, year; hydrogen deployment trends]

While these developments are essential to achieving net-zero goals, they are fundamentally reshaping how energy is generated, delivered, and consumed. A defining characteristic of this evolving system is the sharp increase in variability and uncertainty, which pose significant challenges to maintaining a stable and reliable real-time balance between supply and demand. [For example: wind output in Germany varying by X% in winter; EV charging surges in Norway during cold spells] Variability arises from the intermittent nature of weather-dependent renewables, which fluctuate over timescales ranging from seconds to seasons. Uncertainty further complicates system operation, as it becomes increasingly difficult to predict both renewable output and changing patterns of energy demand – particularly with the growth of electrified heating and transport, distributed energy resources, and more dynamic consumer behaviors. Together, these factors reduce the predictability and controllability of energy systems, rendering traditional operational and planning approaches – based on centralized, dispatchable fossil generation and long-term fuel storage – increasingly inadequate.

Historically, energy systems relied on the flexibility of gas-fired plants, coal reserves, and large-scale fuel storage to absorb demand fluctuations and ensure system stability. In contrast, modern clean energy systems are characterized by high shares of non-dispatchable renewables, electrified and decentralized end-use sectors (e.g., rooftop solar, electric vehicles), and declining levels of dispatchable capacity. These trends diminish the inherent flexibility of conventional systems while amplifying the need for new, responsive forms of flexibility to ensure secure, efficient, and low-carbon operation.

Energy system flexibility refers to the capacity of an energy system to respond to short- and long-term variations in supply and demand while maintaining stability and efficiency. This includes flexibility over multiple timescales – from sub-second grid frequency responses to seasonal energy shifting – and across multiple energy vectors, including electricity, gas, and hydrogen. Enhancing flexibility is essential to support high penetrations of renewables, mitigate system stress, maintain energy security, and reduce the cost of decarbonization as carbon-intensive technologies are phased out.

Flexibility can be delivered through a broad portfolio of technological and systemic solutions. These include supply-side resources such as dispatchable clean generation, cross-border interconnectors, and various forms of energy storage; demand-side options such as industrial load shifting, smart home energy management, and vehicle-to-grid (V2G) integration; and cross-vector approaches such as power-to-gas, hydrogen storage, and electrified heat integration. Increasingly, digital technologies – including real-time sensors, AI-based forecasting, and automated control platforms – are enabling responsive, decentralized flexibility, particularly from distributed assets. Yet the provision of flexibility is not purely a technical matter. Market design and regulatory frameworks must also evolve to properly recognize, value, and incentivize flexibility services, especially where they compete with or defer conventional network investments.

In this Review, we provide a comprehensive assessment of the flexibility technologies and strategies that underpin the functioning of clean energy systems. We structure our discussion around two interconnected domains: electricity system flexibility, and natural gas and hydrogen system flexibility, encompassing both supply-side and demand-side resources. Within this framework, we examine the roles and values of storage, dispatchable generation, interconnection, and demand response, as well as the enabling functions of digitalization and cross-vector integration. We also assess the barriers to broader flexibility deployment, including infrastructure constraints and technology integration challenges. Building on this, we explore how market structures and regulatory frameworks must evolve to support flexibility investment at scale in a cost-effective manner. Finally, we highlight the systemic importance of flexibility for clean energy systems and emphasize the need for interconnected multiple energy vectors to ensure that flexibility is integrated in parallel with decarbonization efforts.

Energy system flexibility [1000]

Fig. 1 | Landscape to clean energy systems. A conceptual diagram showing the transition from centralized, fossil-based energy systems to flexible, multi-vector clean energy systems. Includes: electricity, gas, hydrogen, and thermal networks; embedded flexibility assets (e.g., batteries, electrolysers, heat pumps, EVs, linepack); and flows of energy and control signals (physical and digital).

Fig. 2 | Role and value of flexibility in clean energy systems. A multi-axis plot or schematic showing x-axis: timescale of flexibility; y-axis: services (e.g., balancing, capacity, resilience); markers for where different technologies operate (e.g., batteries, pumped hydro, linepack, EVs). Alternatively, a circular diagram or matrix of flexibility values across time and system levels (consumer → DSO → TSO → cross-border). Technology Landscape for Flexibility: A matrix or layered diagram classifying flexibility technologies by timescale (short-term to seasonal) and domain (supply, demand, cross-vector), highlighting operational range and maturity.

As clean energy systems evolve, the need for flexibility becomes not just a feature of good design, but a core requirement for system stability, efficiency, and decarbonization. Flexibility enables energy systems to respond dynamically to changes in supply and demand, accommodate the variability of renewable energy, and ensure security of supply across diverse timescales and energy vectors. In contrast to legacy systems that relied on dispatchable fossil fuel generation and long-term fuel storage, clean energy systems must deploy more distributed, real-time, and cross-sectoral solutions. This section outlines the conceptual landscape of energy system flexibility, defines its key dimensions, and highlights its functional and systemic value in clean energy transitions.

Dimensions of flexibility

Flexibility in clean energy systems operates across a range of timescales, energy carriers, and system layers – each addressing distinct technical and economic challenges. Understanding these dimensions is essential for designing integrated, resilient, and decarbonized energy systems.

From a temporal perspective, flexibility is needed at multiple timescales. At the shortest scale – ranging from sub-seconds to minutes – energy systems require rapid adjustments to maintain grid frequency and voltage stability. These services, once provided primarily by synchronous generators, are now increasingly supported by fast-response technologies such as battery energy storage systems and grid-forming inverters.

Over longer durations, spanning minutes to hours, flexibility is essential to manage intra-day imbalances caused by forecasting errors or steep changes in net load, especially with high shares of wind and solar generation. Resources such as pumped hydro, flexible gas turbines, and demand-side response are commonly used to meet these needs. On a daily basis, flexibility must accommodate shifts in energy demand and renewable supply over hours to full days. This includes managing evening peaks in electricity demand following solar output declines or smoothing wind variability. Technologies like short-duration battery storage, interconnectors, and smart building loads play a vital role in this space.

At the longest timescales – weeks to seasons – flexibility addresses prolonged mismatches, such as during winter periods with low renewable output and high heating demand. Long-duration solutions, including hydrogen storage, power-to-gas systems, and gas infrastructure with high-pressure linepack, provide deep flexibility for these conditions. For example, power-to-gas systems can store excess electricity as chemical energy for later use.

In addition to operating over time, flexibility spans multiple energy carriers. Electricity systems, which must balance supply and demand instantaneously and have limited inherent storage, are most sensitive to short-term imbalances. In contrast, gas and hydrogen systems offer substantial built-in flexibility, including large-scale storage in salt caverns and the ability to modulate pipeline pressure. Thermal systems, particularly in buildings and industrial processes, can also provide valuable flexibility through thermal storage and load shifting.

Critically, energy system flexibility is no longer confined to individual sectors or technologies. Increasingly, it is achieved through sector coupling – the integration of electricity, gas, hydrogen, and heat networks – to optimize system-wide balancing. This cross-vector coordination enables more effective use of available resources and supports a cost-effective, resilient clean energy transition.

Functional role and value of flexibility

Energy system flexibility plays a vital functional role in ensuring the reliable, efficient, and secure operation of clean energy systems. As the energy transition accelerates, flexibility becomes a foundational enabler – allowing systems to adapt in real time to changing conditions while maintaining balance, avoiding renewable curtailment and load shedding, and reducing infrastructure and operational costs. Its functions extend across operational, economic, and strategic dimensions.

Operationally, flexibility ensures the continuous balance of supply and demand across all timescales. In electricity systems, this includes fast frequency response, short-term ramping to respond to variations in renewable output or demand, and longer-term balancing across hours and days to manage diurnal cycles. Without sufficient flexibility, systems may face reliability risks such as frequency instability, load shedding, or involuntary curtailment of renewable generation. Flexibility also enhances system resilience by supporting black start capabilities, enabling islanding during grid disturbances, and allowing energy systems to recover more rapidly from unexpected events.

Economically, flexibility improves system efficiency and cost-effectiveness. By reducing the need for overbuilt generation and transmission capacity, it enables more efficient use of existing infrastructure. It can also reduce reliance on high-cost peaking plants, lower wholesale price volatility, and enable greater participation of distributed energy resources. For example, demand-side flexibility – through industrial load shifting, electric vehicle charging control, or smart appliances – can flatten demand peaks and reduce system strain during critical periods. On the supply side, technologies like energy storage, interconnectors, and flexible generation can minimize renewable curtailment and optimize resource dispatch.

Flexibility also plays a key role in integrating variable renewable energy, which is increasingly dominant in clean energy systems. High shares of wind and solar introduce both temporal mismatch and uncertainty into generation profiles. Flexibility allows systems to absorb surplus renewable output when it exceeds demand and to supply stored or dispatchable energy when renewable generation is low. In doing so, it enables higher renewable penetration without compromising system stability, and it maximizes the use of zero-marginal-cost energy sources.

Beyond operational and economic benefits, flexibility supports strategic planning by enabling alternative investment pathways that incorporate distributed energy resources and demand-side participation. By deferring or replacing costly upgrades to transmission and distribution networks, flexibility can help regulators and system operators pursue more adaptive and modular planning approaches. Moreover, in future systems characterized by sector coupling and decentralization, flexibility will act as the connective tissue linking technologies, infrastructures, markets, and users into a coherent whole.

Systemic need for flexibility

Flexibility is not just a technical requirement but a systemic necessity for the successful transition to clean energy systems. As fossil fuel-based infrastructure is phased out and replaced by variable renewable generation, decentralized assets, and electrified end-use sectors, the system’s capacity to adapt becomes increasingly central to its overall resilience and performance. Without adequate flexibility, system operators may be forced to curtail renewables, overbuild infrastructure, or rely on expensive peaking resources – undermining both the economic and environmental goals of the energy transition.

Moreover, flexibility supports a more integrated and cross-sectoral energy system, enabling sector coupling and dynamic coordination between electricity, gas, hydrogen, and heat networks. It facilitates more efficient infrastructure investment, accelerates renewable deployment, and enhances consumer participation by allowing demand-side resources to play an active role in system balancing.

In a future characterized by uncertainty – driven by climate impacts, electrification trends, and changing consumption patterns – flexibility offers a buffer against volatility and a tool for proactive system design. It must therefore be treated not as a supplementary service, but as a core pillar of system architecture and planning. The following sections explore the range of technologies and strategies that can provide this flexibility, and the conditions needed to unlock their full potential.

Flexibility technologies [3563]

Fig. 3 | Flexibility technologies in clean energy systems. Supply-side flexibility technologies: clean dispatchable generation, large scale electricity storage, and grid interconnectors. Demand-side flexibility technologies: residential, industrial, commercial, and transport demand-side response. Multi-vector flexibility technologies: power-to-X conversion, linepach and gas network, district heating and thermal storage. [Highlight response speed, operational duration, and technological maturity of each technology; and system-level contributions, including balancing, variability absorption, decarbonization, and resilience.]

Delivering flexibility in clean energy systems requires a diverse and complementary portfolio of technologies that operate across multiple energy vectors, temporal scales, and geographic levels. These technologies differ in response speed, operational duration, and technological maturity, yet collectively contribute to maintaining energy system balance, absorbing variability, and accelerating decarbonization. We categorize these flexibility technologies into three core domains: supply-side resources, demand-side enablers, and multi-vector integration technologies (Fig.3). For each category, we assess their technical functions, system-level impacts, and contributions to the reliable, efficient, and sustainable operation of future clean energy systems.

Supply-side flexibility technologies [1551]

Supply-side flexibility technologies enhance the controllability and responsiveness of energy systems by enabling the adjustment of generation in real time or the injection of stored energy during periods of peak demand or low renewable output. These technologies span a diverse set of solutions, including clean dispatchable generation, large-scale electricity storage, and grid interconnectors.

Clean dispatchable generation [645]

In the pathway to net-zero emissions, it is projected that nearly 70% of global electricity generation by 2050 will come from solar and wind power. While these weather-dependent resources are increasingly cost-competitive and technically viable at scale, their inherent intermittency poses a challenge to real-time system balancing. Clean dispatchable generation technologies – capable of producing electricity on demand with minimal or zero greenhouse gas emissions – can help address this gap. Key candidates include hydropower, clean natural gas, hydrogen-based synthetic fuels, flexible nuclear power, and biomass.

Hydropower is the largest global source of clean dispatchable generation, providing critical flexibility to clean energy systems. As a mature renewable technology, it delivers rapid response capabilities – ramping output within seconds to minutes – and offers operational durations ranging from hours to weeks, depending on reservoir size. Reservoir-based hydropower plants are especially valuable for their ability to adjust generation quickly to match grid demand. Recent innovations in turbine design and digital control systems are further enhancing flexibility to better accommodate variable loads and high shares of intermittent renewables. Compared to run-of-river facilities, flexible hydropower provides more firm capacity by concentrating output during peak hours and supports system balance through operating reserves and frequency regulation.

Natural gas remains a globally important energy source due to its storability, transportability via pipelines or in liquefied form, and suitability for fast-ramping power plants, which can respond within minutes. These characteristics make it well-suited to provide both short-term (intra-day) and seasonal flexibility and can operate at various scales, from local grids to national systems. While cleaner than coal, natural gas still emits substantial quantities of CO2 and methane emissions. Mitigation options include the use of carbon capture, utilization, and storage (CCUS) technologies, which can reduce point-source emissions by up to 90%. Though technically proven, CCUS faces high capital and infrastructure costs, limiting its scale. An alternative is renewable natural gas (RNG), produced from biogas generated via anaerobic digestion or thermal gasification of organic waste. RNG can be upgraded to biomethane and directly injected into existing gas networks as a drop-in fuel, providing low-carbon flexibility without major infrastructure changes.

Hydrogen-based synthetic fuels represent another promising pathway. These fuels are synthesized by combining green hydrogen – produced via electrolysis using renewable electricity – with captured CO2 through methanation or other chemical processes. The resulting products, including synthetic methane, methanol, and e-diesel, are compatible with existing gas turbines and combustion infrastructure, allowing their use without substantial retrofits. They offer dispatchability within minutes to hours and storage durations from days to seasons. When sustainably produced, synthetic fuels provide carbon-neutral, storable energy that bridges electricity, heating, and transport sectors, contributing to sector coupling and grid resilience.

Flexible nuclear power is evolving beyond its traditional role as baseload generation. Advances in control systems and reactor design now allow conventional plants to ramp output within hours, while Small modular reactors (SMRs) are specifically engineered for dynamic operation and modular deployment. SMRs typically target load-following roles with response speeds ranging from tens of minutes to hours and operational durations from daily cycling to baseload. As an early-stage technology, SMRs are under demonstration in several countries but hold high potential as firm, low-carbon capacity. Their ability to integrate with cogeneration applications – such as hydrogen production or district heating – adds further value, enhancing energy system efficiency and multi-vector flexibility.

Biomass-based generation, including solid biomass, biogas, and biomass-fueled combined heat and power (CHP) systems, is inherently dispatchable, with rapid start-up capabilities ranging from seconds (for biogas engines) to hours (for solid biomass boilers). Operational durations vary from sub-hourly frequency regulation to multi-day or seasonal output when dry biomass is stockpiled. Technologically mature and commercially available, flexible biomass can provide grid services, firm capacity, and thermal energy. However, its decarbonization potential hinges on sustainable sourcing and lifecycle emissions accounting. When integrated with thermal storage or district heating, biomass generation supports local energy self-sufficiency, enhances system flexibility, and displaces fossil-based peaking plants.

Large-scale electricity storage [520]

Large-scale electricity storage technologies play a vital role in enhancing the temporal flexibility of clean energy systems. By shifting energy across seconds to days or even seasons, these technologies help absorb variability from renewable generation, stabilize grid operations, and maintain supply-demand balance under high shares of intermittent sources like wind and solar. The primary technologies deployed at scale include short-duration battery energy storage systems (BESS), and long-duration options such as pumped hydro storage (PHS), compressed air energy storage (CAES), and liquid air energy storage (LAES).

Battery energy storage systems (BESS) – primarily based on lithium-ion chemistry – have experienced rapid growth in recent years due to declining costs, modularity deployment, and fast response capabilities, typically on the order of milliseconds to seconds. BESS are best suited for short- to medium-duration storage applications, typically ranging from 1 to 6 hours. They offer high round-trip efficiencies of 85-95%, making them ideal for frequency regulation, voltage support, load shifting, and peak shaving. Co-located with solar and wind farms, batteries help firm renewable output and reduce curtailment, enhancing the grid’s ability to absorb variable energy. While lithium-ion technology is commercially mature, newer chemistries – such as sodium-ion, solid-state, and flow batteries – are emerging to address fire safety risks, resource sustainability, and scalability for longer durations. These advancements are extending the technical functionality and resilience of battery systems in multi-day storage scenarios.

Pumped hydro storage (PHS) is the most mature and widely deployed large-scale storage technology, with global operating capacity exceeding 160 GW. It operates by using surplus electricity to pump water from a lower to an upper reservoir, storing energy as gravitational potential. When needed, the stored water is released to convert the gravitational energy back into electricity through hydraulic turbines, with response times typically under a minute and discharge durations from 10 to over 50 hours. PHS offers round-trip efficiencies of 70-85% and operational lifespans that often exceed 40 years, providing stable, reliable, and cost-effective long-duration storage. Its large energy capacity and ability to deliver both sustained power and ancillary services (e.g., inertia, voltage regulation) make it a cornerstone of system-level reliability in regions with suitable topography. Despite its maturity, PHS deployment is constrained by site availability, environmental impacts, and permitting timelines.

Compressed air energy storage (CAES) and liquid air energy storage (LAES) provide mechanical, long-duration storage options suited for grid-scale balancing. CAES stores compressed air in underground caverns or tanks during periods of low demand and releases it to drive turbines during peak periods. Discharge durations typically range from 8 to 24 hours, with response speeds in the minutes range. CAES is a proven but geographically constrained technology due to its dependence on cavern availability. LAES, by contrast, liquefies air using excess electricity and stores it in insulated tanks. Upon regasification and expansion, the air drives turbines to produce electricity. LAES offers discharge durations from 6 to 20+ hours, response times under 10 minutes, and modular siting flexibility – making it well-suited for urban or industrial settings, especially when integrated with waste heat or cold sources. Together, CAES and LAES represent promising long-duration storage solutions that can complement batteries and pumped hydro in clean energy systems.

Grid interconnectors [322]

Grid interconnectors are high-capacity transmission links – typically using high-voltage alternating current (HVAC) or high-voltage direct current (HVDC) technologies – that physically connect separate electricity grids across regions or countries. They provide critical system-level flexibility by enabling the real-time transfer of electricity across geographic boundaries, facilitating resource sharing, and supporting cross-border balancing of supply and demand. Their response speed typically ranges from seconds to minutes, depending on control and market dispatch systems, and they can operate continuously to shift energy over multi-hour to multi-day periods.

Interconnectors are technologically mature, with decades of successful deployment in Europe, North America, and other interconnected markets. They support high renewable energy penetration by transferring surplus electricity – such as excess wind from coastal regions or solar from high-insolation zones – to areas with concurrent demand or low generation, thus minimizing curtailment and reducing the reliance on fossil-based peaking plants. Their operational duration is effectively unconstrained, as they function as part of the real-time grid backbone.

From a system integration perspective, interconnectors enhance grid stability by enabling synchronized frequency regulation, load balancing, and mutual support during system disturbances or unexpected outages. They also allow countries to pool reserve capacity and access emergency power during periods of stress, improving reliability without the need for redundant domestic infrastructure. This shared flexibility capacity defers investments in new local generation and transmission assets, reducing system costs.

Advanced control technologies – such as dynamic line rating (DLR) and flexible AC transmission systems (FACTS) – are increasingly used to optimize the performance of interconnectors by improving power flow controllability and enabling real-time adaptation to thermal and network constraints. These digital upgrades enhance the value of interconnectors by extracting additional flexibility from existing infrastructure.

Overall, grid interconnectors are a vital enabler of reliable and cost-efficient clean energy systems. They reduce emissions by displacing carbon-intensive generation, facilitate the regional integration of variable renewable energy, and support resilient, multi-national electricity systems capable of withstanding spatial and temporal variability in generation and demand.

Demand-side flexibility technologies [1092]

The growing digitalization and electrification of end-use sectors have transformed consumers from passive energy users into active participants in energy system balancing. Demand-side flexibility, including structured demand-side response (DSR) programs, has emerged as a scalable, distributed, and cost-effective complement to supply-side resources. It allows electrical loads to be adjusted in real time or shifted across hours or days in response to system needs, market signals, or direct control commands from grid operators or aggregators. DSR serves as a key mechanism that enables residential, commercial, industrial, and transport end users to provide grid flexibility, helping to maintain system balance and reliability without the need for additional generation capacity.

Domestic smart appliances [324]

Domestic smart appliances – such as smart thermostats, washing machines, refrigerators, water heaters, and HVAC (heating, ventilation, and air conditioning) systems – are key enablers of domestic demand-side flexibility. When integrated with smart meters and home energy management systems (HEMS), these internet-connected devices can automatically adjust their operation in real time or based on user-defined preferences in response to electricity price signals, grid conditions, or demand response commands. Their response speed is typically fast (seconds to minutes), and their operational duration can range from short (e.g., delaying appliance start by 30 minutes) to several hours, depending on user settings and system needs. By shifting demand away from peak periods or toward times of surplus renewable generation – such as midday rooftop solar output – smart appliances help flatten household demand profiles, reduce stress on distribution networks, and improve system-wide efficiency.

Smart meter-enabled time-of-use (ToU) tariffs form the foundation for this flexibility, allowing consumers to benefit financially by shifting consumption to off-peak hours. However, broader uptake remains limited, and more dynamic pricing structures – such as critical peak pricing and real-time tariffs – may be required to fully unlock the value of residential DSR, particularly during high-stress or low-inertia grid conditions.

Despite their potential, domestic smart appliances currently face challenges in participating in local flexibility markets to support distribution grid constraint management. These markets are predominantly designed for commercial and industrial users, and concerns remain among distribution network operators (DNOs) about the consistency and reliability of aggregated residential DSR. Overcoming these challenges will require advances in standardization, interoperability, cybersecurity, and automation, along with appropriate incentives and consumer protections.

As smart meter penetration increases and demand-side market structures mature, domestic smart appliances are well positioned to become a core element of distributed flexibility portfolios. Their ability to deliver fast, decentralized, and consumer-friendly demand modulation supports system balancing, reduces reliance on peaking plants, and defers costly infrastructure investments – all while helping integrate higher shares of renewable energy in a reliable and cost-effective manner.

Industrial and commercial demand response [307]

Industrial and commercial demand response (ICDR) refers to the strategic adjustment of electricity consumption by non-residential energy users – such as manufacturers, commercial buildings, public sector facilities, and small-to-medium enterprises – in response to price signals, grid conditions, or contractual obligations. These users often have high electricity intensity and large controllable loads, enabling substantial flexibility contributions from a limited number of participants.

ICDR typically operates with response speeds ranging from seconds to minutes for ancillary services (e.g., frequency response) and hours for load shifting. Operational durations vary from short-term reductions (minutes to hours) to extended shifts across daily cycles, depending on process flexibility and business constraints. Flexibility is often embedded in industrial processes (e.g., electrolysis, aluminum smelting, cold storage) and large-scale building systems (e.g., HVAC, lighting, refrigeration). With the rapid expansion of AI and digital services, data centers are emerging as a significant source of demand-side flexibility due to their large, controllable electricity loads and potential to shift non-time-critical computing tasks in response to grid signals. Automated control systems and real-time monitoring are essential to ensure fast, reliable adjustments with minimal disruption to core operations. However, participation is contingent on operational feasibility, infrastructure readiness, and the ability to capture economic value.

ICDR contributes to energy system balance and decarbonization by enabling load shifting to align with periods of high renewable output, reducing reliance on fossil-based peaking plants, and mitigating system stress during high-demand periods. For example, pre-cooling in cold storage facilities or scheduling non-critical manufacturing processes during off-peak hours can flatten demand curves and absorb renewable variability. Advanced ICDR programs extend into real-time balancing markets, providing ancillary services such as frequency regulation, voltage support, and spinning reserve. Fast-response capabilities – enabled by digital controls – are increasingly common in modern industrial sites. Over longer durations, ICDR supports capacity adequacy and can participate in capacity markets, particularly valuable during seasonal peaks or extreme weather conditions. Furthermore, ICDR provides geographically targeted flexibility that can alleviate local network congestion and defer grid reinforcements. Its scalability, reliability, and range of technical functions make it a cornerstone of a flexible, efficient, and decarbonized energy system.

Smart and bidirectional charging [339]

Electric vehicles (EVs) spend approximately 95% of their time parked, representing a substantial untapped resource for grid flexibility. Through smart charging (V1G) and bidirectional charging technologies – collectively referred to as vehicle-to-everything (V2X) – EVs can transition from passive energy consumers to active, distributed energy storage resources, that enhance the flexibility, reliability, and sustainability of clean energy systems.

Smart charging (V1G) is the most mature and widely adopted EV flexibility solution. It enables unidirectional energy flow from the grid to the vehicle, with charging schedules optimized based on grid prices, user preferences, or renewable availability. Typically operating over short-to-medium durations (2-8 hours), smart charging can respond within minutes and is particularly effective in shifting demand away from system peak periods or toward surplus renewable output. This demand-side management reduces strain on local networks and supports grid balancing, all without compromising user mobility.

Bidirectional charging (V2X) further enhances flexibility by enabling EVs to discharge electricity back to the grid or into local devices. In vehicle-to-grid (V2G) applications, EVs act as distributed storage assets capable of responding in real time (seconds to minutes) to system needs, offering services such as peak shaving, frequency regulation, and capacity adequacy. In vehicle-to-home (V2H) or vehicle-to-building (V2B) configurations, EVs improve energy self-sufficiency by powering household or facility loads during high-price or low-renewable periods. Operational durations for V2X typically range from a few minutes to several hours, depending on battery size, usage patterns, and system needs.

With appropriate digital infrastructure, EVs can participate in ancillary service markets, real-time price response, and local flexibility schemes. Their distributed nature and mobility provide geographic flexibility, enabling them to support grid constraints or renewable integration wherever they are connected.

As the market for V2X-capable vehicles, chargers, and supporting standards matures, EVs are increasingly recognized as "storage on wheels" – offering scalable, fast-response flexibility that contributes to system reliability, decarbonization, and consumer empowerment. While bidirectional charging technologies are still emerging and require supportive regulation, interoperability, and business models, their integration will be critical to maximizing the role of transport electrification in clean energy transitions.

Multi-energy flexibility technologies [931]

Multi-energy flexibility technologies – also referred to cross-vector or sector-coupling solutions – enable the coordinated conversion, storage, and exchange of energy across multiple carriers, including electricity, gas (e.g., hydrogen), and thermal energy. These technologies enhance overall system flexibility by allowing surplus energy from one vector to be shifted, transformed, or stored to meet demand in another, improving efficiency, reliability, and decarbonization. At their core, multi-energy systems (MES) are designed to manage different energy vectors in an integrated manner, leveraging intersectoral synergies to optimize resource utilization. The portfolio of MES flexibility solutions includes power-to-X conversions, gas networks and linepack storage, and district heating with thermal energy storage that coordinate cross-vector flows at the community, regional, or national level.

Power-to-X conversion [294]

Power-to-X (P2X) technologies offer significant MES flexibility by converting surplus electricity – typically from variable renewables – into alternative energy carriers such as gas, heat, hydrogen, and synthetic fuels. These conversions provide valuable pathways for sector coupling, enabling electricity to support decarbonization in transport, industry, and heating, while also contributing to grid balancing, energy storage, and system resilience.

One of the most mature P2X pathways is power-to-gas (P2G), which uses electrolysis to convert electricity into green hydrogen. This hydrogen can be stored (with durations from hours to seasons), injected into gas networks, or used directly in industrial processes and fuel cells. It can also be synthesized into renewable fuels like methane or ammonia, enhancing dispatchability and transportability. P2G provides medium- to long-duration energy storage and facilitates deep sectoral integration by linking electricity and gas systems. It helps relieve grid congestion during periods of renewable overgeneration and delivers low-carbon dispatchable fuel when needed.

Power-to-heat (P2H) technologies convert electricity into thermal energy using resistance heaters, heat pumps, or electrode boilers. The thermal energy can be used immediately in buildings or industry or stored in thermal buffers (e.g., hot water tanks) and district heating networks for later use. With fast response times (seconds to minutes) and short to medium storage durations (hours to days), P2H supports demand-side flexibility, helps manage peak loads, and enhances renewable integration, especially when paired with thermal energy storage (TES).

Power-to-liquids (P2L) technologies involves the production of synthetic liquid fuels – such as methanol or e-kerosene – by combining renewable electricity with captured CO2. These fuels offer long-duration, high-energy-density storage, are compatible with existing transport infrastructure, and are critical for aviation, shipping, and heavy industry. Though less mature and costlier than P2G or P2H, P2L technologies provide dispatchable, low-carbon fuel options in sectors where electrification is impractical.

Gas networks and linepack storage [195]

Gas networks, particularly using linepack storage, offer an important source of MES flexibility by leveraging the compressibility of gas in transmission pipelines. By modulating pipeline pressure, operators can temporarily store or release significant volumes of gas, providing intra-day and short-term balancing capabilities without additional infrastructure. Linepack typically offers response times of minutes to hours, making it valuable for managing short-term demand fluctuations or supply variability, especially during peak heating periods or renewable output dips.

When integrated with centralized gas storage such as salt caverns, aquifers, or depleted reservoirs, the system gains medium- to long-duration storage capabilities, extending balancing over weeks to seasons. This combined infrastructure enables a robust gas system flexibility portfolio, supporting both base and peak loads with high reliability.

As gas grids decarbonize – via the integration of green hydrogen and renewable natural gas (RNG) – linepack is expected to play an increasingly critical role in storing and transporting low-carbon gases, thus enhancing sector coupling between electricity and gas networks. Its technical maturity, embedded availability, and low marginal cost make it an efficient and cost-effective buffering mechanism that complements P2G and hydrogen electrolysis, smoothing mismatches between intermittent generation and end-use demand in clean energy systems.

District heating with thermal energy storage [311]

District heating systems, when integrated with thermal energy storage (TES), provide a highly scalable and cost-effective source of flexibility by decoupling heat production from consumption. These systems distribute heat – often sourced from low-carbon heat technologies such as large-scale heat pumps, geothermal energy, or industrial waste heat – through insulated pipelines to serve residential, commercial, or industrial users. When coupled with TES units like hot water tanks or molten salt reservoirs, district heating systems can absorb surplus renewable electricity via P2H conversion during periods of low demand or high generation, and store the heat for later use. TES systems typically have response speeds ranging from minutes to hours and can offer medium- to long-duration flexibility, spanning hours to several days depending on the storage technology and system scale.

This flexibility enables the shifting of electrical loads, mitigation of peak electricity demand, and greater utilization of renewable energy by absorbing surplus output from wind and solar generation. Technologically, both district heating and TES are well-established and widely deployed, with high energy density and long operational lifetimes, making them well-suited for delivering medium- to long-duration thermal flexibility. Their economic viability and reliability make them especially attractive for urban and industrial energy systems. As a core component of broader MES, district heating with TES strengthens electricity-heat sector coupling, enhances local energy self-sufficiency, and supports a resilient, efficient, and low-carbon energy transition.

Moreover, district heating networks can incorporate Combined Heat and Power (CHP) units, which simultaneously generate electricity and heat from a single fuel source. CHP plants can dynamically adjust their heat-to-power output ratio in response to electricity price signals or grid needs, enhancing both thermal and electrical flexibility. When combined with TES, CHP units can operate during off-peak hours and store thermal energy for later use. This synergy improves system adaptability, reduces renewable curtailment, and supports deep decarbonization by maximizing fuel efficiency and enabling cross-sectoral flexibility.

Flexibility strategies [2000]

Fig. 4 | Global deployment of flexible technologies – current maps and future projections

Fig. 5 | Global deployment of flexible strategies – market and regulation

Deployment and projections [600]

Supply-side flexibility technologies

Clean dispatchable generation: hydropower, clean natural gas, renewable natural gas (RNG), hydrogen-based synthetic fuels, small modular reactors (SMRs), biomass-based generation.

Global Power Plants Map (hydro, biomass): https://atlas.co/maps/global-power-plants-map/. Building an open guide to the world’s energy system: https://globalenergymonitor.org/

Carbon Storage Locations - World View: https://public.tableau.com/app/profile/gabe.dewitt/viz/CarbonStorageLocations-WorldView/Dashboard8. https://www.netl.doe.gov/carbon-management/carbon-storage/worldwide-ccs-database. CCUS Projects Database (missing XY): https://www.iea.org/data-and-statistics/data-product/ccus-projects-database

Hydrogen Production and Infrastructure Projects Database: https://www.iea.org/data-and-statistics/data-product/hydrogen-production-and-infrastructure-projects-database. A regularly updated compendium of global low-carbon hydrogen projects and their status: https://www.thehydrogenmap.com/

Small Modular Reactor (SMR) Global Tracker (online only): https://world-nuclear.org/information-library/current-and-future-generation/small-modular-reactor-smr-global-tracker

Large-scale electricity storage: battery energy storage systems (BESS), pumped hydro storage (PHS), compressed air energy storage (CAES) and liquid air energy storage (LAES). Storage Lab: https://www.storage-lab.com/data. Global Greenfield Pumped Hydro Energy Storage Atlas: https://re100.eng.anu.edu.au/global/. Pumped Storage Tracking Tool: https://www.hydropower.org/hydropower-pumped-storage-tool. The DOE Global Energy Storage Database: https://sandia.gov/ess-ssl/gesdb/public/projects.html.

Grid interconnectors. Net electricity imports, 2024. Net electricity imports are calculated as electricity imports minus exports. Countries with positive values are net importers ofelectricity; negative values are net exporters. Measured in terawatt-hours (country to missing XY): https://ourworldindata.org/grapher/net-electricity-imports. Net electricity imports as a share of electricity demand, 2024. Net electricity imports are calculated as electricity imports minus exports. This is given as a share of a country's electricitydemand. Countries with positive values are net importers of electricity; negative values are net exporters: https://ourworldindata.org/grapher/electricity-imports-share-demand. Energy imports and exports, 2015. Energy trade, measured as the percentage of energy use. Positive values indicate a country orregion is a net importer of energy. Negative numbers indicate a country or region is a netexporter: https://ourworldindata.org/grapher/energy-imports-and-exports-energy-use?country=OWID_EU27~WB_EAP~WB_ECA~GBR~CHN~USA

Demand-side flexibility technologies

Domestic smart appliances. Smart home (market revenues): https://www.statista.com/outlook/cmo/smart-home/worldwide

Industrial and commercial demand response. All_DataCenters: https://public.tableau.com/app/profile/harish4340/viz/All_DataCenters/ByCity2. New All Data Centers: https://public.tableau.com/app/profile/harish4340/viz/NewAllDataCenters/Sheet44. SeperateDataCenters: https://public.tableau.com/app/profile/harish4340/viz/SeperateDataCenters/America. IEA Interactive map of data centre hubs alongside associated power and digital infrastructure: https://www.iea.org/data-and-statistics/data-tools/energy-and-ai-observatory?tab=Energy+for+AI. Data center: https://www.datacentermap.com/datacenters/. Global Data Centers Locations (6346): https://www.datacenters.com/locations. Find Your Next Data Center: https://baxtel.com/map?lat=1.5631940186722204e-13&lng=37.878281816803366&distance=8910438.735813458. Data center world map: https://map.datacente.rs/. HVAC (residential, commercial, industrial): https://www.marketdataforecast.com/market-reports/hvac-equipment-market. HVAC Analysis: https://public.tableau.com/app/profile/dave.kellermanns/viz/HVACAnalysis/SensorData

Smart and bidirectional charging. Global EV Data Explorer: https://www.iea.org/data-and-statistics/data-tools/global-ev-data-explorer. 601,183 charging stations for electric cars in the world: https://www.electromaps.com/en/charging-stations. Electric car stocks, 2024. Car stocks represent the number of cars that are in use. It is the balance of cumulative sales over time and the number ofcars that have been retired or taken off the road. Electric cars include fully battery-electric vehicles and plug-in hybrids: https://ourworldindata.org/grapher/electric-car-stocks?tab=map. Share of cars currently in use that are electric, 2024: Electric cars include fully battery-electric and plug-in hybrids: https://ourworldindata.org/grapher/share-car-stocks-electric?tab=map. Tracking global data on electric vehicles: https://ourworldindata.org/electric-car-sales. Open Charge Map - EV Charging Locations: https://public.tableau.com/app/profile/open.charge.map/viz/OpenChargeMap-EVChargingLocations/Map. NYC EV Charging Station Locations: https://public.tableau.com/app/profile/catherine.zhou8560/viz/NYCEVChargingStationLocations/Sheet1

Multi-energy flexibility technologies

Power-to-X conversion. Global PtX Atlas: https://devkopsys.de/ptx-atlas/. https://maps.iee.fraunhofer.de/ptx-atlas/. Global GIS-based potential analysis and cost assessment of Power-to-X fuels in 2050

Gas networks and linepack storage. Global Gas Infrastructure Tracker: https://globalenergymonitor.org/projects/global-gas-infrastructure-tracker/tracker/

District heating with thermal energy storage. Heat Roadmap Europe: Identifying strategic heat synergy regions. UK heat network map: https://data.barbour-abi.com/smart-map/repd/desnz/?type=heat_network. District Energy in North America: https://www.districtenergy.org/resources/resources/system-maps. Thermal energy storage: https://sandia.gov/ess-ssl/gesdb/public/projects.html

Market and Regulations [600]

Countries with a sustainable consumption and production national action plan: This metric measures whether countries have implemented policies as part of a sustainable consumption and production plan. This doesnot measure the quality or ambition of these plans, only whether a country has one: https://ourworldindata.org/grapher/country-scp-plan

When will countries phase out coal power? This measures pledges to phase out coal from the electricity mix: https://ourworldindata.org/grapher/coal-phase-out-timeline

Aggregators and digital platforms [147]

To enable broad participation in flexibility markets across sectors, aggregators and digital platforms are emerging as key intermediaries that coordinate and manage distributed energy resources (DERs) such as residential batteries, EVs, heat pumps, and smart appliances. By pooling these decentralized assets, aggregators form virtual power plants (VPPs) capable of delivering fast-response (seconds to minutes), short- to medium-duration flexibility across multiple sectors.

These platforms use real-time data, AI-driven algorithms, and standardized communication protocols to optimize DER dispatch in response to market prices, grid conditions, or control signals. VPPs can participate in wholesale markets, balancing services, and local flexibility schemes – offering grid operators a scalable and decentralized alternative to conventional peaking plants.

While still evolving, the technological maturity of aggregator platforms is rapidly advancing. Their deployment enhances grid stability, improves renewable integration, and unlocks new revenue opportunities for consumers – supporting a more resilient, efficient, and participatory clean energy system.

Energy hubs and integrated energy systems [165]

Energy hubs are localized, multi-vector infrastructures – often located in campuses, communities, or industrial parks – that manage the production, conversion, storage, and consumption of multiple energy carriers, including electricity, gas, and heat. By leveraging real-time data, digital controls, and coordinated storage assets, these hubs optimize local energy flows, enable demand-side flexibility, and facilitate the integration of variable renewable energy at the distribution level. Their ability to shift loads across vectors and time enhances system reliability, reduces peak demand, and supports decarbonization in decentralized energy settings.

At broader scales, integrated energy systems operate across regional or national boundaries, coordinating cross-sectoral energy flows using advanced forecasting, automated dispatch, and market-based mechanisms. These systems enable cross-vector flexibility – shifting energy in time (short- to long-term), form (electricity, thermal, gas), and space (local to national) – to absorb variability, maintain system balance, and ensure efficient use of resources. As such, they form the backbone of a resilient, interoperable, and sustainable clean energy future, supporting deep decarbonization across power, heat, transport, and industry.

Unlock flexibility of demand and supply. Flexibility is vital in a system with more variable renewables. There are large opportunities to increase flexibility in both demand and supply, across residential and commercial applications, and in industry. However, flexibility is not currently valued in full and faces multiple barriers. Markets and processes must be redesigned, and data and digital approaches embraced, enabling infrastructure must be delivered and barriers unblocked.

Natural gas remains a globally important energy source due to its storability, transportability via pipelines or in liquefied form, and suitability for use in fast-ramping power plants. These characteristics make it well-suited to provide both short-term and seasonal flexibility to energy systems. However, despite being less carbon-intensive than coal, natural gas still emits substantial quantities of CO2 and methane emissions. Efforts to reduce its climate impact include carbon capture, utilization, and storage (CCUS) technologies, involving three key steps: post-combustion capture, reused in various industrial processes, and compressed and injected into deep geological formations. as seen in recent developments such as six announced CCUS-equipped gas power projects in the United States and the Net Zero Teesside Power project in the United Kingdom, scheduled for commercial operation by 2027. Another pathway is the use of renewable natural gas (RNG), produced from biogas generated via anaerobic digestion or thermal gasification of organic waste. Once upgraded to biomethane, the resulting high-purity RNG can be injected directly into existing natural gas pipelines, making it a drop-in replacement without requiring major infrastructure modifications. with over 300 operational facilities in North America and more than 80 connected to the grid in the UK.

3.1 Global Deployment Trends and Future Projections (to 2050)

- Overview: Briefly introduce how flexibility technologies are scaling globally, driven by electrification, decarbonization, and digitalization.

- By Technology Category:

  - Supply-side:

    - Clean dispatchable generation: Role of flexible gas, nuclear, and biomass; phase-out vs. retrofitting projections.

    - Large-scale electricity storage: Global installed BESS and pumped hydro; forecasted GW capacity by 2030/2040/2050.

    - Grid interconnectors: Growth of cross-border HVDC projects (e.g., Europe, ASEAN), increasing interconnection capacity.

  - Demand-side:

    - Domestic smart appliances: Smart meter penetration, projected household DSR participation.

    - ICDR: Sectoral uptake, potential MW flexibility unlocked, trends in automation.

    - Smart & bidirectional EV charging: EV fleet growth, share of smart/V2X-enabled chargers by 2030 and 2050.

    - Aggregators/VPPs: Number of aggregators, share of DERs managed, scaling potential.

  - Multi-energy systems:

    - P2X (P2G, P2H, P2L): Deployment of electrolyzers, sector integration forecasts.

    - Gas networks & linepack: Status of hydrogen blending, evolution of hybrid gas-electric systems.

    - District heating & TES, CHP: Deployment in cold-climate regions, projected heat storage volume.

    - Energy hubs: Growth of local integrated systems in cities and industrial clusters.

3.2 Barriers to Broader Deployment

- Technology-specific challenges:

  - High capital costs, infrastructure constraints (e.g., pumped hydro, CAES).

  - Interoperability, cybersecurity (smart appliances, V2X).

  - Feedstock and sustainability constraints (biomass).

  - Low returns or uncertain revenue models (aggregators, TES, CHP).

- Cross-cutting barriers:

  - Lack of market access for distributed assets.

  - Institutional and planning inertia (especially for multi-vector integration).

  - Limited consumer awareness and engagement.

3.3 Market and Regulatory Evolution for Flexibility Investment

- Redesigning markets to value flexibility:

  - Move from energy-only markets to flexibility-focused capacity and ancillary service markets.

  - Time- and locational-value-based pricing (e.g., dynamic ToU tariffs, flexibility zones).

- Enabling distributed and aggregated flexibility:

  - Open local flexibility markets, dynamic procurement by DSOs.

  - Fair compensation for DSR, aggregator participation frameworks.

- Regulatory frameworks and policy enablers:

  - Standardization and interoperability rules.

  - Mandates for smart metering, V2G readiness.

  - Incentives or mandates for multi-vector coordination (e.g., national flexibility targets).

- International collaboration and knowledge sharing:

  - Harmonizing flexibility valuation across regions.

  - Joint infrastructure investments (e.g., interconnectors, hydrogen corridors).

Summary and future perspectives [500]

Investing in flexibility is a no-regrets decision as it delivers material net savings of up to £16.7bn/yr across all net zero scenarios analysed in 2050

Flexibility supports a net zero energy system to cope with dark, cold and windless days in winter

Embedding flexibility in zero carbon heat and transport solutions will help to reduce their system impact and costs making their decarbonisation economically more feasible

Flexibility is deployed more locally in 2050 and delivers significant value nationally 

Reaching net zero by 2050 whilst meeting security of supply requires unprecedented build-out across the energy system

Pushing the energy system to go beyond zero carbon has material cost and infrastructure implications

Carbon negative technologies have an important role in helping to meet the net zero target in 2050

Any single heating solution dominant future has a significant cost and infrastructure impact on the energy system in 2050

The use of hydrogen across the energy system brings carbon and cost benefits and requires a portfolio of production methods and availability of CCS infrastructure

Flexibility should be integrated into enabling infrastructure including low carbon heat and transport solutions from the start

Consumer engagement on flexibility beyond just commercial value is a critical aspect to scaling up flexibility technology deployment

An evolving regulatory environment, combined with potentially low financial gains in the long term, creates challenges for business model development

A smart and flexible system can only be enabled by digitalisation of the energy system

Continued efforts for new technology development and innovation focused on cross vector integration is important to have them ready in time